Name: 	 Class Period: 	 Date:

# CALCULATING RAIN GARDEN SIZE

## Step 1: Calculate area of the roof.

Examine the shape and size of the roof illustrated on the next page.

The formula needed to calculate the roof area is:

  - Area of a rectangle = length x width

(Repeat calculation for each roof segment and add together.)

## Step 2: Find the sizing factor for the soil type the rain garden will be built on.

- The sizing factor adjusts for the permeability of the soil.

|  | Soil Type |  |  |
| --- | --- | --- | --- |
|  | Sandy | Silty | Clayey |
| Sizing Factor | 0.13 | 0.3 | 0.35 |

Data source: Wisconsin DNR Rain Garden Design Manual

## Step 3: Multiply the roof area by the sizing factor to calculate the rain garden area needed.

### Example

Building is 30 feet wide and 50 feet long. Soils are silty.

- Area of roof is 30 feet x 50 feet = 1500 square feet

- Sizing factor for silty soils is 0.3

- 1500 square feet x 0.3 sizing factor = 450 square foot rain garden is needed

Answer: 450-square-foot rain garden is needed to capture runoff from this roof if the soils are silty.

Note: A 450-square-foot rain garden could be 15 feet by 30 feet, 18 feet by 25 feet, or any other shape that has an area of 450 square feet.

Your Turn

My calculations for the area of a rain garden needed for this roof & sandy soil.

My calculations for the area of the roof.

The school building and its roof is shaped like the letter T. The top portion of the T is 20 feet wide by 50 feet long. The vertical portion of the T is 30 feet wide by 60 feet long. (This is the same school roof used in the Calculating Runoff Amounts off a Roof handout). The soils at the school are sandy. Calculate the area of a rain garden that would be needed to capture runoff from this school roof. Show your work.